SECTION 107326 – MANUFACTURED WALKWAY COVERS

PART 1 - GENERAL

  - Related Documents

- Drawings and general provisions of contract, including General & Supplementary Conditions and Division 1 Specification Sections, shall apply to work specified in this section.

  - General Description of Work

- Work in this section shall include design, fabrication, and installation of complete welded, extruded aluminum canopy system. All work shall be in complete accordance with the drawings and this specification.

  - References

- Aluminum Design Manual 2000, Specifications & Guidelines for Aluminum Structures.

- ASCE 7, Minimum Design Loads for Buildings and Other Structures.

- American Architectural Manufactures Association (AAMA).

- American Society for Testing and Materials (ASTM).

  - Related Sections

- Concrete Work – Section 03300

- Masonry Work – Section 04200

- Miscellaneous Metals – Section 05500

- Flashing and Sheet Metal – Section 07600

- Sealants – Section 07900

  - Submittals

- Product Data: Submit manufacture’s product information, specifications and installation instructions for components and accessories.

- Shop Drawings: Submit complete erection drawings showing attachment system, column and beam framing, transverse cross sections, covering and trim details, and option installation details to clearly indicate proper assembly of components. Detail shop drawings shall be submitted, sealed by a State Registered Structural Engineer.

- Design Data: Design calculations bearing the seal of a State Registered Structural Engineer. Calculations shall verify that framing design will safely resist wind loading requirements, snow loading requirements, and seismic requirements of the current International Building Code (IBC), as well as complying with ANSI/ASCE 7.

- Samples for Verification: For each type of exposed finish provide a physical color sample of the manufacturer’s standard colors.

- Closeout Data: O&M Manual containing product maintenance and final executed warranty.

  - Quality Assurance

- Codes and Standards: Comply with provisions of the following except as otherwise indicated: Current International Building Code (IBC). American Welding Society (AWS) standards for structural aluminum welding.

- Manufacturer: Obtain aluminum covered walkway system from only one (1) manufacturer, although several may be indicated as offering products complying with requirements.

- Installer Qualifications: Firm with not less than three (3) years experience in installation of aluminum walkway covers of type, quality and installation methods similar to work of this section.

- Field Measurements: Take field measurements prior to preparation of shop drawings and fabrication where possible, to ensure proper fitting work.

- Coordination: Coordinate work of this section with work of other sections which interface with covered walkway system (sidewalk, curbs, building fascias, etc.)

  - Warranty

- Product and Installation Warranty: Provide manufacture’s standard one-year warranty that shall include, but not limited to, coverage for structural failure, deuteriation of metals beyond normal weathering, sealant failure, and water tightness beginning the day of Substantial Completion of Installation.

- Finish Warranty: Provide manufacturer’s standard one-year warranty for factory finishing beginning the day of Substantial Completion of Installation.

PART 2 – PRODUCT

  - Manufactures

- Contract documents are based on products manufactured by:

Tennessee Valley Metals Inc., East Coast TVM (Carolina’s) (843)346-3885

4709 Fraley Drive,

Timmonsville, SC 29161

sales@tvmetals.com, www.ectvm.com

- Interested manufactures will be considered for substitution only when the following conditions are met: Complete details, including sizes of all members and structural calculations showing loads applied in accordance with the applicable building code must be submitted to for review. Submit complete details with structural properties (moment of inertia, section modules, modulus of elasticity, etc.) for all proposed sections (bents, columns, decking, and other structural members).

  - Materials

- Aluminum Extrusions: All sections shall be extruded aluminum 6063 alloy, heat treated to T-6 temper.

- Two Coat Kynar - Fluoropolymer Two-Coat System. Manufacturer’s standard two-coat thermocured system consisting of specially formulated inhibitive primer and fluoropolymer color topcoat containing not less than 70 percent polyvinylidene fluoride resin by weight complying with AAMA 2605.

  - Components

- Columns: Columns shall be radius cornered tubular extrusions of size specified by manufacturer’s structural engineer with cutout and internal diverter for drainage where indicated. Circular downspout opening in column is not acceptable.

- Beams: Beams shall be open-top tubular extrusions of size specified by manufacturer’s structural engineer, top edges thickened for strength and designed to receive deck members in self-flashing manner. Structural ties shall be installed in tops of all beams.

- Deck: Deck shall be extruded self-flashing sections interlocking into a composite unit.

- Fascia: Fascia shall be manufacture’s standard shape. Size as indicated on drawings.

- Flashing: Flashing shall be .032 aluminum (min.).  All through-wall flashing is completed by others.

  - Fabrication

- Drainage: Water shall drain internally from deck to beams to columns, for discharge out to rain diverters at or below ground level as indicated on architectural drawings.

- Deck Construction: Deck shall be manufactured of extruded modules that interlock in a self-flashing manner. Interlocking joints shall be positively fastened as spaced in shop drawings creating a monolithic structural unit capable of developing the full strength of the sections. The fastening must have minimum shear strength of 350 lbs. each. Deck shall be assembled with sufficient camber to offset dead load deflection.

PART 3 – EXECUTION

  - Preparation

- Erection shall be performed after all concrete, masonry, and roofing work in the vicinity is complete and cleaned.

  - Installation

- Column Sleeves: Column sleeves (Styrofoam block outs) or anchor bolts (if required) shall be furnished by Tennessee Valley Metals, East Coast TVM or approved equal and installed by the General Contractor.

- Erection: Protective cover shall be erected true to line, level and plumb at height indicated in drawings, with surfaces free from distortion or other defects in appearance.

  - Cleaning

- All protective cover components shall be cleaned promptly after installation.  Remove protective films from members. Clean canopy of dirt, grease, handprints, and other blemishes. Leave area in a neat, clean, and acceptable condition.

  - Protection

- Extreme care shall be taken to protect materials during and after installation.